CAPSTONE PROJECT

TRAFFIC-LIGHT CONTROLLER + MOD-N COUNTER

AIM

To design and simulate a synchronous traffic-light controller using a finite-state machine (FSM) and a reusable Mod-N counter in Verilog HDL, and to verify the phase sequence and timing behavior using the Vivado Design Suite.

SOFTWARE USED

Xilinx Vivado Design Suite

THEORY

A traffic-light controller is a sequential digital system that changes its outputs according to a defined sequence of states. In this project, the North-South (NS) and East-West (EW) directions are controlled so that only one direction can have a green light at a time. Each green phase is followed by a yellow phase, after which the other direction receives green.

A finite-state machine (FSM) stores the current phase in a state register. A timer counts clock cycles in that phase and causes a transition when the selected phase duration is reached. The design uses synchronous state updates and non-blocking assignments in clocked always blocks.

A Mod-N counter counts from 0 to N−1 and then returns to 0. It is useful for generating periodic timing intervals. For a clock frequency f_clk and a required interval T, the approximate number of ticks is N = T × f_clk.

DESIGN SPECIFICATIONS

The design has four traffic phases: NS Green, NS Yellow, EW Green, and EW Yellow. The sequence repeats continuously after reset is released. Green and yellow durations are configurable through parameters.

Use a single rising-edge clock and an active-high synchronous reset.

Reset the FSM to NS Green, with EW Red active.

Follow the sequence NS Green → NS Yellow → EW Green → EW Yellow → NS Green.

Never assert NS Green and EW Green simultaneously.

Provide a parameterized Mod-N counter with enable and terminal-count output.

Verify behavior in simulation and inspect the waveform before attempting hardware implementation.

INPUTS AND OUTPUTS

Traffic-light controller inputs: clk, rst. Outputs: ns_red, ns_yellow, ns_green, ew_red, ew_yellow, ew_green. All light outputs are active-high.

Mod-N counter inputs: clk, rst, enable. Outputs: count and terminal.

TRUTH TABLE / STATE TABLE

The following table shows the expected light outputs in each FSM state. A value of 1 means that lamp is ON.

| State | NS Red | NS Yellow | NS Green | EW Red | EW Yellow | EW Green |
| --- | --- | --- | --- | --- | --- | --- |
| NS Green | 0 | 0 | 1 | 1 | 0 | 0 |
| NS Yellow | 0 | 1 | 0 | 1 | 0 | 0 |
| EW Green | 1 | 0 | 0 | 0 | 0 | 1 |
| EW Yellow | 1 | 0 | 0 | 0 | 1 | 0 |

TIMING AND STATE TRANSITIONS

The FSM transitions after the configured duration of each phase. In the included testbench, GREEN_TICKS is set to 5 and YELLOW_TICKS is set to 2 to make the transitions easy to observe in simulation. These are demonstration values, not real-road timing recommendations.

NS Green → NS Yellow when the green timer expires.

NS Yellow → EW Green when the yellow timer expires.

EW Green → EW Yellow when the green timer expires.

EW Yellow → NS Green when the yellow timer expires.

VERILOG HDL SOURCE CODE — MOD-N COUNTER

`timescale 1ns/1ps module mod_n_counter #( parameter integer N = 10, parameter integer WIDTH = (N <= 2) ? 1 : $clog2(N) ) ( input wire clk, input wire rst, input wire enable, output reg [WIDTH-1:0] count, output wire terminal ); assign terminal = (count == N-1); always @(posedge clk) begin if (rst) count <= {WIDTH{1'b0}}; else if (enable) begin if (terminal) count <= {WIDTH{1'b0}}; else count <= count + 1'b1; end end endmodule

VERILOG HDL SOURCE CODE — TRAFFIC-LIGHT CONTROLLER

`timescale 1ns/1ps module traffic_light_controller #( parameter integer GREEN_TICKS = 10, parameter integer YELLOW_TICKS = 3, parameter integer COUNT_WIDTH = 16 ) ( input wire clk, input wire rst, output reg ns_red, output reg ns_yellow, output reg ns_green, output reg ew_red, output reg ew_yellow, output reg ew_green ); localparam [1:0] NS_GREEN = 2'd0; localparam [1:0] NS_YELLOW = 2'd1; localparam [1:0] EW_GREEN = 2'd2; localparam [1:0] EW_YELLOW = 2'd3; reg [1:0] state; reg [COUNT_WIDTH-1:0] timer; reg [COUNT_WIDTH-1:0] phase_limit; always @* begin case (state) NS_GREEN: phase_limit = GREEN_TICKS - 1; NS_YELLOW: phase_limit = YELLOW_TICKS - 1; EW_GREEN: phase_limit = GREEN_TICKS - 1; EW_YELLOW: phase_limit = YELLOW_TICKS - 1; default: phase_limit = GREEN_TICKS - 1; endcase end always @(posedge clk) begin if (rst) begin state <= NS_GREEN; timer <= 0; end else begin if (timer >= phase_limit) begin timer <= 0; case (state) NS_GREEN: state <= NS_YELLOW; NS_YELLOW: state <= EW_GREEN; EW_GREEN: state <= EW_YELLOW; EW_YELLOW: state <= NS_GREEN; default: state <= NS_GREEN; endcase end else begin timer <= timer + 1'b1; end end end always @* begin ns_red = 0; ns_yellow = 0; ns_green = 0; ew_red = 0; ew_yellow = 0; ew_green = 0; case (state) NS_GREEN: begin ns_green = 1; ew_red = 1; end NS_YELLOW: begin ns_yellow = 1; ew_red = 1; end EW_GREEN: begin ew_green = 1; ns_red = 1; end EW_YELLOW: begin ew_yellow = 1; ns_red = 1; end default: begin ns_red = 1; ew_red = 1; end endcase end endmodule

TESTBENCH CODE

`timescale 1ns/1ps module tb_traffic_light_controller; reg clk = 0; reg rst = 1; wire ns_red, ns_yellow, ns_green; wire ew_red, ew_yellow, ew_green; traffic_light_controller #( .GREEN_TICKS(5), .YELLOW_TICKS(2), .COUNT_WIDTH(8) ) dut ( .clk(clk), .rst(rst), .ns_red(ns_red), .ns_yellow(ns_yellow), .ns_green(ns_green), .ew_red(ew_red), .ew_yellow(ew_yellow), .ew_green(ew_green) ); always #5 clk = ~clk; initial begin $dumpfile("traffic_light.vcd"); $dumpvars(0, tb_traffic_light_controller); $display("time NS(RYG) EW(RYG)"); $monitor("%4t %b%b%b %b%b%b", $time, ns_red, ns_yellow, ns_green, ew_red, ew_yellow, ew_green); repeat (2) @(posedge clk); rst = 0; repeat (30) @(posedge clk); $finish; end endmodule

SIMULATION RESULTS / WAVEFORM

RTL SCHEMATIC